# ⭐ 1. NETWORK DEVICES (routers, switches, load balancers, SAN fabric)

### 🔹 What they are

Devices that move traffic inside the data center / cloud:

- routing

- switching

- load balancing

- connecting storage networks (SAN fabric)

SAN here = network side of storage, not disks.

### 🔹 Common real-world devices

Routers / Switches

- Cisco (Catalyst, Nexus)

- Juniper

- Arista

- Dell / HP Aruba

Load balancers

- F5 BIG-IP

- NGINX / HAProxy

- Citrix ADC

SAN Fabric

- Cisco MDS

- Brocade Fibre Channel

### 🔹 How Elastic Agent collects data (Fleet)

You typically use integrations like:

- Cisco

- Juniper

- NGINX

- F5

- Syslog

- SNMP

Most network devices send logs via syslog → Elastic Agent receives them.

Metrics (CPU, interfaces, bandwidth) usually come via SNMP integrations.

### ✔ Data we collect

#### Logs

Examples:

- interface up/down

- link flaps

- routing changes

- power / hardware failure

- authentication failures

Example:

Interface GigabitEthernet0/1 changed state to down

#### Metrics

Key ones:

| Metric | Why important |
| --- | --- |
| Bandwidth in/out | capacity |
| Packet drops / errors | congestion |
| CPU / Memory | device health |
| Latency | network slowness |
| Discards | quality issues |

### 📊 Typical dashboards show

- Top bandwidth interfaces

- Network utilization trends

- Link failure history

- Packet loss / drops

- Device CPU / memory

- Alerts for flaps or overloads

👉 Symptom chain example

Packet drops ↑ → latency ↑ → app appears “slow”.

# ⭐ 2. SECURITY DEVICES (firewalls, IDS/IPS, endpoint security, proxies)

### 🔹 What they are

Devices & platforms that protect the environment:

- block malicious traffic

- detect intrusions

- protect endpoints

- control internet access

- audit activity

### 🔹 Widely used devices / tools

Firewalls

- Palo Alto

- Fortinet FortiGate

- Cisco ASA / FirePower

- CheckPoint

- Sophos

IDS / IPS

- Suricata

- Snort

- Zeek

Proxies / Web gateways

- Bluecoat

- Zscaler

- Squid

Endpoint & security platforms

- Elastic Security

- CrowdStrike

- SentinelOne

- Wazuh

### 🔹 How Elastic Agent collects data

Fleet integrations (very common):

- Palo Alto

- Fortinet

- Cisco Secure

- Zeek / Suricata

- Elastic Defend

- Syslog ingestion

- Proxy integrations

Firewalls usually send logs to Elastic via syslog.

Endpoint security often uses Elastic Defend directly on hosts.

### ✔ Data collected

#### Logs (MOST important)

- allowed / denied traffic

- threat detections

- VPN login success/failure

- malware alerts

- policy changes

- user activity auditing

Example:

action=deny src=10.10.1.20 dst=8.8.8.8 port=443 rule=block_external

#### Metrics

(secondary, but useful)

- active sessions

- blocked vs allowed traffic

- CPU / memory

- connection load

### 📊 Typical dashboards show

- Top attackers / IPs

- Top blocked destinations

- Threat categories

- VPN login failures

- Suspicious spikes in traffic

- Audit logs / policy change history

👉 These dashboards are alert-driven.

A typical story:

sudden spike in blocked outbound traffic → malware → investigate host → isolate.

# ⭐ 3. STORAGE DEVICES (actual disk arrays, NAS/SAN storage)

### 🔹 What they are

Systems where application data actually lives.

This is different from SAN network devices.

### 🔹 Widely used storage systems

Enterprise arrays

- Dell EMC (Unity / PowerStore / VNX)

- NetApp ONTAP

- HPE 3PAR / Nimble

- Hitachi Vantara

- IBM FlashSystem

Hyperconverged

- VMware vSAN

- Nutanix

Cloud

- AWS EBS / S3

- Azure Managed Disks

- GCP Persistent Disks

### 🔹 How Elastic Agent collects storage data

Typical integrations:

- NetApp

- VMware

- SNMP

- Custom API ingestion

- Log integrations from storage controllers

Many storage vendors expose:

- logs via syslog

- metrics via API / SNMP

Elastic Agent pulls them through Fleet.

### ✔ Data collected

#### Logs

- disk failures

- RAID rebuild

- controller failure

- snapshot jobs

- replication failures

#### Metrics (critical for performance)

| Metric | Why it matters |
| --- | --- |
| IOPS | workload intensity |
| Latency | user experience |
| Throughput | data rate |
| Disk usage % | capacity planning |
| Queue depth | bottlenecks |
| Cache hit ratio | performance efficiency |

### 📊 Dashboards show

- storage capacity trends

- latency spikes

- IOPS trends per volume

- failed disks / degraded arrays

- replication health

- snapshot schedule success/failure

👉 Many “application slow” issues end up being storage latency issues.

# 🧩 How all of this fits together in ELK (Fleet world)

With Elastic Agent via Fleet, you'll typically see indices like:

logs-*

metrics-*

network-*

security-*

apm-*

Data flow:

Device → Syslog/SNMP/API → Elastic Agent Integration → Fleet → Elasticsearch → Dashboards

Correlations become powerful:

- network congestion → app latency

- firewall blocking → user login failure

- storage latency → database slow

- SAN switch error → storage performance degraded

input {

udp {

port => 514

type => "syslog"

}

}

input {

udp {

port => 514

type => "syslog"

}

}